IUBAT-International University of Business Agriculture and Technology

# Network Design for InspireITSolutions with Multiple Offices in Bangladesh

Submitted To

Mahedi Hasan

Lecturer

Dept. of CSE

Submitted By

Name:Fatema Taj Mim

ID: 23103406

Section:H

Course Code: CSC 465

Department: BCSE

Semester: Spring – 2026

Date Of Submission: 23rdMay 2026

Contents

- Introduction	2

- NetworkDesignandDiagram	2

  - OverallArchitecture	.............................	2

  - DhakaOffice(Headquarters).........................	4

  - ChattogramOffice	..............................	4

  - RemoteDevelopers..............................	4

- IPAddressingPlan	4

  - AddressingStrategy	.............................	4

  - SubnetAllocationTable...........................	5

  - JustificationforIPv4/Dual-StackChoice.................	5

- Inter-OfficeConnectivity	5

  - RecommendedMethod:Site-to-SiteIPsecVPNoverBroadband.....	5

  - VPNConfigurationSteps	..........................	5

- SecurityMeasures	6

  - SecurityArchitectureOverview.......................	7

  - FirewallRules.................................	7

  - IntrusionPreventionSystem(IPS)	.....................	7

  - VPNforRemoteDevelopers.........................	7

  - AuthenticationMethods...........................	8

  - AdditionalSecurityControls.........................	8

- WirelessNetworkSetup	8

  - AccessPointPlacement	...........................	8

  - SSIDConfiguration..............................	8

- OptimizationPlan	9

  - QualityofService(QoS)...........................	9

  - BandwidthManagement...........................	9

  - NetworkMonitoringTools	10

  - ScalabilityPlanning	10

- SimulationResults(Optional)	10

- Conclusion	10

- KeyPerformance Area(KPA) Outcomesand Justification	11

References	13

# Introduction

This report presents a comprehensive network design for InspireITSolutions, a mid-sized software company operating from two offices in Dhaka (Headquarters) and Chat-togram, Bangladesh. The objective is to build a secure, high-performance, and scalable networkinfrastructurethatsupportsdailybusinessoperations,centralizeddatamanage-ment, inter-office communication, and secure remote access for off-site developers.

Thedesignaddressesthefollowingkeyrequirements:

- DepartmentaltrafficsegregationusingVLANs.

- Securesite-to-siteVPNconnectivitybetweenDhakaandChattogram.

- RemoteVPNaccessfor10developerslocatedacrossBangladesh.

- CentralizeddatastorageintheDhakaDataCenter,accessiblefromChattogram.

- WirelessnetworkingwithseparateofficeandguestSSIDs.

- QualityofService(QoS)policiesforcriticalbusinesstraffic.

- Scalabilitytoaccommodatea20%deviceincreasewithintwoyears.

# NetworkDesignandDiagram

## OverallArchitecture

The network follows a hierarchical three-layer model (Core, Distribution, and Access layers)adaptedtothecompany’stwo-sitetopology.Figure1showsthehigh-levellogical topology.

DhakaHeadquarters

AdminSwitch

VLAN10

DevSwitch

VLAN20

TestSwitch

VLAN30

DataCenter

VLAN40(Servers)

CoreSwitch/Router

(L3Aggregation)

NGFWFirewall

(DhakaEdge)

Site-to-SiteVPN

RemoteDevs

(SSLVPNClients)

ClientVPN

Internet/WAN

(IPsec&SSLTunnels)

Site-to-SiteVPN

BranchRouter

(ChattogramEdge)

Dist.Switch

(L2/L3)

SalesSwitSchupportSwitLchocalNAS

VLAN60

VLAN70

VLAN80

ChattogramBranchOffice

Figure1:LogicalNetworkTopology–InspireITSolutions

## DhakaOffice(Headquarters)

The Dhaka HQ hosts the core infrastructure.Three access-layer switches serve the Ad-ministrative, Development, and Testing departments. A distribution-layer switch aggre-gatesdepartmenttrafficandconnectstothecorerouter.Adedicatedfirewall(CiscoASA orFortinet)sitsbetweenthecorerouterandtheinternetuplink,andalsoterminatesthe site-to-site VPN tunnel.

| tableheader |  |  |  |  |
| --- | --- | --- | --- | --- |
| Administrative | 20 | VLAN | 10 | Accessswitch,Wi-FiAP |
| rowgrayDevelopment | 50 | VLAN | 20 | Accessswitch,Wi-FiAP |
| Testing | 30 | VLAN | 30 | Accessswitch,Wi-FiAP |
| rowgrayDataCenter | Servers | VLAN40 Applicationservers,fileservers,NA |  |  |
| Management(OOB) | – | VLAN | 99 | Router,switches(management) |
| GuestWi-Fi | Variable | VLAN | 50 | IsolatedguestSSID |

Table1:DhakaOffice–DeviceSummary

S

## ChattogramOffice

A branch router connects via the VPN tunnel to Dhaka.Two access-layer switches serve the Sales/Marketing and Customer Support departments.A local storage server holds departmental data and replicates critical data back to Dhaka nightly.

Table2:ChattogramOffice–DeviceSummary

| tableheader |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sales&Marketing | 20 | VLAN | 60 | Accessswitch, Wi-FiAP |  |
| rowgrayCustomer Support | 30 | VLAN | 70 | Accessswitch, Wi-FiAP |  |
| LocalStorage | 1server | VLAN | 80 | NAS/storageserver |  |
| rowgrayGuestWi-Fi | Variable | VLAN | 90 | IsolatedguestSSID |  |

## RemoteDevelopers

Ten remote developers connect over the internet to the Dhaka office VPN concentrator using client-to-site VPN (IPsec/IKEv2 or SSL/TLS). Each session is authenticated with a username,password,andaone-timepassword(OTP).Remotedevelopertrafficisplaced in a dedicated subnet (192.168.50.0/28).

# IPAddressingPlan

## AddressingStrategy

IPv4 with dual-stack readiness is chosen.Pure IPv4 (RFC 1918 private space) is used internallybecauseallcurrentdevices,networkappliances,andISPsinBangladeshreliably support it. IPv6 prefix delegation is configured on edge routers for future readiness, but internal VLANs remain IPv4 for the foreseeable term.

Subnettingapproach:Startingblockis192.168.0.0/16.EachVLANreceivesa/24or /26 depending on device count, with headroom for the mandated 20% growth.

## SubnetAllocationTable

Table3:CompleteIPAddressingPlan

| tableheader |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DhakaOffice |  |  |  |  |  |  |  |
| Administrative | 10 | 192.168.10.0/26 | 255.255.255.192 | 20 | 62 | 192.168.10.1 |  |
| rowgrayDevelopment | 20 | 192.168.20.0/25 | 255.255.255.128 | 50 | 126 | 192.168.20.1 |  |
| Testing | 30 | 192.168.30.0/26 | 255.255.255.192 | 30 | 62 | 192.168.30.1 |  |
| rowgrayDataCenter | 40 | 192.168.40.0/27 | 255.255.255.224 | 10 | 30 | 192.168.40.1 |  |
| GuestWi-Fi(DK) | 50 | 192.168.50.0/25 | 255.255.255.128 | Var | 126 | 192.168.50.1 |  |
| rowgrayManagement | 99 | 192.168.99.0/28 | 255.255.255.240 | – | 14 | 192.168.99.1 |  |
| ChattogramOffice |  |  |  |  |  |  |  |
| Sales&Marketing | 60 | 192.168.60.0/26 | 255.255.255.192 | 20 | 62 | 192.168.60.1 |  |
| rowgrayCustomerSupport | 70 | 192.168.70.0/26 | 255.255.255.192 | 30 | 62 | 192.168.70.1 |  |
| LocalStorage | 80 | 192.168.80.0/28 | 255.255.255.240 | 5 | 14 | 192.168.80.1 |  |
| rowgrayGuestWi-Fi(CTG) | 90 | 192.168.90.0/25 | 255.255.255.128 | Var | 126 | 192.168.90.1 |  |
| Inter-OfficeandRemote |  |  |  |  |  |  |  |
| VPNTunnel (S2S) | – | 10.10.10.0/30 | 255.255.255.252 | 2 | 2 | 10.10.10.1 |  |
| rowgrayRemoteDevelopers | – | 192.168.100.0/28 | 255.255.255.240 | 10 | 14 | 192.168.100.1 |  |

## JustificationforIPv4/Dual-StackChoice

    - Compatibility:All existing equipment and ISP services in Bangladesh are IPv4-native.

    - Simplicity:RFC1918privateaddressingiswellunderstoodandeasytomaintain.

    - Dual-stack readiness:Edge routers are configured with IPv6 prefix delegation (e.g., 2001:db8::/32 in production) so the network can adopt IPv6 incrementally without a forklift upgrade.

    - Scalability:A /16 parent block provides thousands of available subnets for future growth.

# Inter-OfficeConnectivity

## RecommendedMethod:Site-to-SiteIPsecVPNoverBroadband

A site-to-site IPsecVPN over dedicated broadband fiber links is recommended. This provides an encrypted, cost-effective, and high-performance WAN connection without the expense of a private MPLS circuit.

Comparisonofoptions:

Table4:Inter-OfficeConnectivityOptions

| tableheader |  |
| --- | --- |
| Site-to-SiteVPN Lowcost, encrypted,flexible ISP | DependentonISPqua |
| rowgrayMPLSLeasedLine GuaranteedSLA,lowlatency | VeryexpensiveinBD |
| SD-WAN Multi-linkfailover,QoSaware | Addedlicensingco |

## VPNConfigurationSteps

- ISPUplinks:Provisiondedicatedfiberbroadbandatbothsiteswithstaticpublic IPs.

er

lity	Ch

Backu

st	Future

- Phase1(IKEv2):ConfigureISAKMPpolicy:AES-256encryption,SHA-256 integrity,DHGroup14(2048-bit),lifetime86400s.

- Phase2(IPsec):ESPwithAES-256-GCM,PFSGroup14,lifetime3600s.

- Pre-sharedKeys/Certificates:UseX.509certificatesfromaninternalPKI forstrongerauthentication.

- TunnelInterface:Bindthetunneltoaloopbackordedicatedinterface(e.g., tunnel0) and route the remote subnet over it.

- Routing:StaticroutesorBGPoverthetunnel:Dhakarouteradvertises192.168.10–40.0/x; Chattogram router advertises 192.168.60–80.0/x.

!Phase1Policy

cryptoisakmppolicy10 encraes256

hashsha256 authenticationpre-share group14

lifetime86400

cryptoisakmpkeyYOUR_PSKaddress<CTG_PUBLIC_IP>

!Phase2TransformSet

cryptoipsectransform-setTS_AES256esp-aes256esp-sha256-hmac modetunnel

!CryptoMap

cryptomapVPN_MAP10ipsec-isakmp setpeer<CTG_PUBLIC_IP>

settransform-setTS_AES256 setpfsgroup14

matchaddressVPN_ACL

!ACL--traffictoencrypt

ipaccess-listextendedVPN_ACL

permitip192.168.0.00.0.255.255192.168.60.00.0.3.255

!ApplytoWANinterface interfaceGigabitEthernet0/0

cryptomapVPN_MAP

Failover:Asecondary4G/LTElinkateachsiteactsasabackupWANwith tracked floating static routes.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

Listing1:CiscoIOS–Site-to-SiteIPsecVPN(DhakaRouter,simplified)

# SecurityMeasures

## SecurityArchitectureOverview

Securityisimplementedinadefense-in-depthlayeredmodelcoveringperimeter,net-work, endpoint, and management planes.

## FirewallRules

Astateful next-generation firewall(NGFW)(e.g., Cisco Firepower, Fortinet For-tiGate)isdeployedateachsite’sinternetedge.

    - Inboundrules:Denyallbydefault;allowonlyHTTPS(443),SSH(22from management VLAN only), and VPN (UDP 500, 4500, ESP) from known IPs.

    - Outbound rules:Allow HTTP/HTTPS for general browsing; block peer-to-peer and non-business ports.

    - Inter-VLANrules:OnlypermittedtrafficflowsbetweenVLANs(e.g.,allVLANs

→Data Center VLAN 40 on defined application ports; development VLAN cannot reach customer support VLAN directly).

    - GuestVLANisolation:GuestVLANs(50,90)haveinternetaccessonly;blocked from all internal VLANs.

## IntrusionPreventionSystem(IPS)

An inline IPS module (integrated into the NGFW or deployed as a dedicated appliance) monitorstrafficforknownattacksignatures,protocolanomalies,andzero-daybehavior using:

    - Signature-baseddetection(updateddailyfromvendorthreatintelligence).

    - Anomaly-baseddetectionforunusualtrafficvolumesorportscans.

    - Automatic blockingofconfirmedthreats;alerts senttotheSIEM.

## VPNforRemoteDevelopers

    - Protocol:SSL/TLSVPN(OpenVPNorCiscoAnyConnect)onport443,which is rarely blocked by ISPs.

    - Authentication:Username +password +TOTP(Time-basedOne-TimePass-wordviaGoogleAuthenticatororMicrosoftAuthenticator).

    - Splittunneling:Disabled–alldevelopertrafficroutesthroughtheDhakaVPN gateway to enforce policy inspection.

    - IPassignment:Remotedevelopersreceiveaddressesfromthe192.168.100.0/28 pool; ACLs restrict access to development servers and repositories only.

## AuthenticationMethods

Table5:AuthenticationMethodsbyUseCase

| tableheader |  |  |
| --- | --- | --- |
| Wired/wirelessLANusers | 802.1XwithActiveDirectory | RADIUS(Fre |
| rowgrayRemotedeveloperVPN | Username+Password+TOTP | RADIUS |
| Site-to-siteVPN | X.509certificates | Intern |
| rowgrayNetworkdevicemanagement | SSH+localadmin+AAA | TA |
| GuestWi-Fi | Captiveportal(weblogin) | Splas |

eRADIUS/

+GoogleAu al PKI (Wind CACS+ (Cisc h page + em

## AdditionalSecurityControls

    - Portsecurity:MACaddresslimitingonaccessports(max2MACperport);unused ports shut down.

    - DHCPsnooping:PreventsrogueDHCPserversonallaccessVLANs.

    - DynamicARPInspection(DAI):BlocksARPspoofingattacks.

    - IPSourceGuard:ValidatessourceIPsagainstDHCPbindings.

    - SIEM:Centralized log collection (e.g., Splunk orELKStack) in the Dhaka DataCenteraggregatesfirewall,IPS,andswitchlogs.

    - Endpointprotection:Antivirus/EDR on all workstations (e.g., CrowdStrike or Microsoft Defender).

# WirelessNetworkSetup

## AccessPointPlacement

Wi-Fi6(802.11ax)accesspointsaredeployedforhigh-densitycoverage.APsareceiling-mountedat approximately every 15–20m for adequate overlap.

Table6:AccessPointPlacementPlan

| tableheader |  |  |
| --- | --- | --- |
| DK–Administrative | 2 | 802.11ax Lowdensity; oneAP |
| rowgrayDK–Development | 4 | 802.11ax Highdensity;dual-band, |
| DK–Testing | 3 | 802.11ax Mediumdensit |
| rowgrayDK–DataCenter | 0 | N/A Wired-only(serv |
| DK–Common/lobby | 1 | 802.11ax GuestSSIDon |
| rowgrayCTG–Sales&Mktg | 2 | 802.11ax Openplan;2APsfor |
| CTG–CustomerSupport | 3 | 802.11ax Call-centerlayout;dens |
| rowgrayCTG–Common/lobby | 1 | 802.11ax GuestSSIDon |

## SSIDConfiguration

EachofficebroadcaststwoSSIDs:

per wing MU-MIMO

y ers) ly

coverage

ecoverage ly

Table7:SSIDConfiguration

| tableheader |  |  |  |
| --- | --- | --- | --- |
| InspireIT-Corp-DK | 5GHz | 10/20/30 | WPA3-Enterprise(802.1X) |
| rowgrayInspireIT-Guest-DK 2.4GHz |  | 50 WPA3-Personal+captiveportal |  |
| InspireIT-Corp-CTG | 5GHz | 60/70 | WPA3-Enterprise(802.1X) |
| rowgrayInspireIT-Guest-CTG 2.4GHz |  | 90 WPA3-Personal+captiveportal |  |

    - WPA3-Enterprise:Authenticates corporate users via RADIUS linked to Active Directory.

    - Guest isolation:Guest SSIDs are mapped to isolated VLANs with firewall rules blockingaccesstoallinternalnetworks.

    - Bandsteering:Capableclientsaresteeredto5GHztoreducecongestionon

2.4GHz.

    - Management:All APs are managed centrally through a wireless controller (e.g.,CiscoWLCorUbiquitiUniFiController)hostedintheDhakaDataCenter.

# OptimizationPlan

Acc

Internal Interne

Internal+

Interne

## QualityofService(QoS)

QoSisimplementedend-to-end(accessswitch→distribution→WAN)usingDSCP marking and queuing.

Table8:QoSTrafficClassification

| tableheader |  |  |
| --- | --- | --- |
| VoIP/VideoConferencing | EF(46) | Priority 30%–strictprio |
| rowgrayDevelopmentTools(Git,CI/CD) AF31(26) |  | Class1 25%–guarante |
| FileSharing/Backup | AF11(10) | Class2 20%–besteffo |
| rowgrayGeneralBrowsing | BE(0) | Default Remainingbandw |
| GuestWi-Fi | BE(0) | Policed Cappedat10Mbps |

## BandwidthManagement

    - Uplink:Minimum200Mbpsdedicatedfiberateachsite(1Gbpsrecommendedfor Dhaka HQ).

    - Trafficshaping:Policingappliedtoguestandpersonalstreamingtraffic.

    - WAN optimization:Deduplication and compression on the site-to-site VPN (e.g., Cisco WAAS or Silver Peak).

    - Loadbalancing:IfdualISPlinksareprovisioned,ECMPloadbalancingdis-tributes traffic.

rity edrt idth

peruser

## NetworkMonitoringTools

Table9:RecommendedMonitoring Tools

| tableheader |  |  |
| --- | --- | --- |
| PRTGNetworkMonitor Bandwidth,uptime,SNMPpolling |  | SaaSoron-prem; |
| rowgrayZabbix | Performancemetrics,thresholds | Open-source;integrate |
| Wireshark /tcpdump | Packetcaptureandanalysis | Forad-hoctr |
| rowgrayNetFlow/sFlow | Trafficflow analysis | Exportedfromrouter |
| Splunk/ELKStack | LogaggregationandSIEM | Correlates security |

## ScalabilityPlanning

    - All subnets are sizedwith atleast20% headroomabove currentdevicecounts.

    - Stackableswitchesareusedattheaccesslayersoadditionalunitscanbeadded without rewiring.

    - The DHCP scope for each VLAN is pre-configured to accommodate the projected growth.

    - ThewirelesscontrollerlicenseisprocuredfortheprojectedmaximumAPcount (current + 30%).

    - TheDataCenterrackspaceisplannedwith4Uoffreespaceperyearforadditional servers.

# SimulationResults(Optional)

dashboards &s with SNMP oubleshootins to collector events across

The following tests were performed using Cisco Packet Tracer.The .pkt file is submitted separately.

- Intra-office connectivity:Ping tests between devices within each VLAN returned 0% packet loss.Inter-VLAN routing via the Layer-3 switch correctly forwarded packets between authorised VLANs and dropped unauthorised traffic per ACL.

- Inter-office communication:End-to-end ping across theVPNtunnel (Dhaka 192.168.10.10→Chattogram192.168.60.10)succeededwithaverageRTTof8ms. File transfer to the Dhaka Data Center from Chattogram was verified usingFTP simulation.

- Remote developer access:A remote PC configured with VPN client software successfullyauthenticatedandreceivedanIPfromthe192.168.100.0/28pool.Ac-cess to the development server (192.168.20.100) was confirmed; access to the ad-ministrativeVLANwasblockedbyfirewallACL,confirmingcorrectsegmentation.

# Conclusion

Thisreporthaspresentedacomplete,production-gradenetworkdesignforInspireIT SolutionscoveringboththeDhakaHeadquartersandtheChattogrambranchoffice.The

keyoutcomesofthedesignare:

- Segmentation:Eight departmental VLANs isolate traffic, reducing the broadcast domain and limiting the blast radius of any security incident.

- Security:A defense-in-depth model combines NGFW, IPS, 802.1X, MFA, and site-to-siteIPsecVPNtoprotect allcommunicationpaths.

- Connectivity:Thesite-to-siteVPNoverfiberbroadbandprovidesanencrypted, high-bandwidthlinkbetweenofficesatafractionofthecostofaleasedline.

- Remote Access:SSL/TLS VPN with MFA enables the 10 remote developers to securely access only the resources they require.

- Wireless:WPA3-Enterprise corporate SSIDs and isolated guest SSIDs at both locations ensure both usability and security.

- Optimization:DSCP-based QoS prioritizes voice, video, and development traffic, while NetFlow and PRTG provide continuous visibility into network health.

- Scalability:The subnetting scheme, stackable switching, and modular wireless controller design accommodate the projected 20% device growth without architec-tural changes.

ThenetworkdesignmeetsallstatedrequirementsofInspireITSolutionsandprovidesa solid foundation for future expansion, including potential migration to SD-WAN and full IPv6 deployment.

# KeyPerformanceArea(KPA)OutcomesandJustification

ToensurethenetworkalignsstrictlywiththebusinessobjectivesofInspireITSolutions, the design satisfies the following Key Performance Areas (KPAs):

- Security & Compliance:Justification:The implementation of a defense-in-depth strategyusingstatefulNGFWs,strictVLANisolation,802.1Xauthentication,and AES-256 encrypted VPNs (both Site-to-Site and Client-to-Site) guarantees robust data confidentiality and protects internal infrastructure from external threat vectors.

- HighAvailability&Reliability:Justification:Failover mechanics via secondary 4G/LTE WAN links, combined with dynamic routing and solid IPsec tunneling, mini-mizepotentialdowntimebetweentheDhakaandChattogramoffices,ensuringcontin-uous access to the centralized Data Center.

- Scalability & Future-Readiness:Justification:A modular, hierarchical three-layer architecturepairedwithagenerousIPaddressingscheme(maintaining20%minimum headroompersubnet)anddual-stackIPv4/IPv6edgereadinessensuresseamlessscal-ing over the next several years without requiring costly forklift upgrades.

- Performance&Optimization:Justification:ByactivelyleveragingDSCP-basedQoS queuing models, critical applications such as VoIP and CI/CD development flows

are securely prioritized over best-effort and guest network traffic, effectively eliminating operational bottlenecks.

- Manageability& Operations:Justification:Establishing an Out-of-Band (OOB) management VLAN, alongside centralized logging (SIEM) and network monitoring solutions(likePRTGorZabbix),drasticallysimplifiesadministrativeoverheadwhile delivering complete visibility to the IT team.

# References

- Tanenbaum,A.S.,&Wetherall,D.J.(2011).ComputerNetworks(5thed.).Pearson.

- Forouzan,B.A.(2013).DataCommunicationsandNetworking(5thed.).McGraw-Hill.

- CiscoSystems.	(2024).	CiscoIOSSecurityConfigurationGuide.	Retrievedfrom

https://www.cisco.com/c/en/us/td/docs/ios/

- RFC4301–SecurityArchitecturefortheInternetProtocol(IPsec).IETF.

- RFC4303–IPEncapsulatingSecurityPayload (ESP).IETF.

- IEEE802.11ax–HighEfficiencyWirelessLAN(Wi-Fi6).IEEEStandardsAssocia-tion.

- RFC1918–AddressAllocationforPrivateInternets.IETF.